INDEX

| S.No | Programs | Teacher’s Sign |
| --- | --- | --- |
| 1. | WAP to calculate 1’s complement of 8 bit number |  |
| 2. | WAP to calculate 2’s complement of 8 bit number. |  |
| 3. | WAP to find the sum of two 8 bit numbers. |  |
| 4. | WAP to subtraction of two 8 bit numbers. |  |
| 5. | WAP to find the largest among two numbers. |  |
| 6. | WAP to find the largest number in an array. |  |
| 7. | WAP to copy block of data from one location to another. |  |
| 8. | Count the number of 1’s in an 8 bit number. |  |
| 9. | WAP for linear search. |  |
| 10. | WAP to count total even numbers in a series of ten numbers. |  |
| 11. | WAP for multiplication of two 8 bit numbers. |  |
| 12. | WAP for division of two 8 bit numbers. |  |
| 13. | WAP to calculate the sum of a series. |  |

CALCULATE 1’S COMPLEMENT OF A 8 BIT NUMBER.

LDA C050

CMA

STA C051

HLT

# ORG C050

# DB 96H

## OUTPUT:-

# PROGRAM 2:- WRITE A PROGRAM TO CALCULATE 2’S COMPLEMENT OF A 8 BIT NUMBER.

LDA C050

CMA

INR A

STA C051

HLT

# ORG C050

# DB 96H

## OUTPUT:-

# FIND SUM OF TWO  8 BIT NUMBER.

LXI H,C050

MOV A,M

INX H

ADD M

STA C052

HLT

# ORG C050

# DB 55H,66H

OUTPUT:-

# PROGRAM 4:- WRITE A PROGRAM TO CALCULATE SUBTRACTION OF TWO  8 BIT NUMBER.

LXI H,C050

MOV A,M

INX H

SUB M

INX H

MOV M,A

HLT

# ORG C050

# DB 95H,65H OUTPUT:

# FIND L RGEST AMONG TWO NUMBERS.

MVI A,34H

MVI B,25H

MOV C,A

CMP B

JNC LOOP

MOV A,B

STA C052

LOOP: MOV A,C

STA C053 HLT

## OUTPUT:

# FIND L RGEST NUMBER IN AN ARRAY.

MVI C,05

LXI H,2501

MOV A,M

DCR C

LOOP :INX H

CMP M

JNC AHEAD

MOV A,M

AHEAD: DCR C

JNZ LOOP

STA 2600

HLT

#ORG 2501

#DB 03,08,55H,12H,06

## OUTPUT:-

COPY	 BLOCK OF DATA FROM ONE LOCATION TO ANOTHER.

# BEGIN 0000H

LXI H,C000H

LXI SP,B000H

MOV B,M

MOV C,M

INX H

SAVE:

MVI D,00H

MOV E,M

PUSH D

DCR C

INX H

JNZ SAVE

MOV C,B

MOV D,M

INX H

MOV E,M

MOV A,E

ADD C

DCR A

MOV E,A

XCHG COPY:

POP D

MOV M,E

DCR C

DCX H

JNZ COPY

HLT

## OUTPUT:-

8	COUNT THE NUMBER OF 1’S

# IN A 8 BIT NUMBER.

MVI B,00 MVI C,08

LDA 3000

RAR

JNC 200C

INR B

DCR C

JNZ 2007 MOV A,B STA 3001

HLT

#ORG3000H

#DB 05

## OUTPUT:-

# PROGRAM 9:- WRITE A PROGRAM FOR LINEAR SEARCH.

LXI H,2040

MOV B,M

LXI H,2050

MVI D,0A

NEXT:    MOV A,M

CMP B

JZ STOP

INX H

DCR D

JNZ NEXT

LXI H,2060

MVI M,00

HLT

STOP:    LXI H,2060

MVI M,01

HLT

#ORG 2040

#DB 02

#ORG 2050

#DB 03,05,02,10H,12H,02,03,01,09,08

## OUTPUT:-

# PROGRAM 10:- WRITE A PROGRAM TO COUNT TOTAL EVEN NUMBERS IN A SERIES OF 10 NUMBERS.

2000:      LXI H,204F

2003:      MVI C,00

2005:      MVI D,0A

2007:      INX H

2008:      MOV A,M

2009:      ANI 01

200B:      JNZ 200F

200E:      INR C

200F:      DCR D

2010:      JNZ 2007

2013:      MOV A,C

2014:      STA 3050

2017:      HLT

#ORG 2050

#DB 02,03,08,01,07,04,0A,3B,05,06 OUTPUT:-

# PROGRAM 11:- WRITE A PROGRAM FOR MULTIPLICATION OF TWO 8 BIT NUMBERS.

# ORG 2000H

# BEGIN 2000H

LHLD 2501

XCHG

LDA 2503

LXI H,0000

MVI C,08

LOOP:    DAD H

RAL

JNC AHEAD

DAD D

AHEAD: DCR C

JNZ LOOP

SHLD 2504

HLT

# ORG 2501H

# DB 84H,00H,56H

## OUTPUT:-

# PROGRAM 12:- WRITE A PROGRAM FOR DIVISION OF TWO 8 BIT NUMBERS.

# ORG 2000H

# BEGIN 2000H

LHLD 2501

LDA 2503

MOV B,A

MVI C,08

LOOP:    DAD H

MOV A,H

SUB B

JC AHEAD

MOV H,A

INR L

AHEAD:	DCR C

JNZ LOOP

SHLD 2504

HLT

# ORG 2501H

// LSB OF DIVIDEND , MSB OF DIVIDEND , DIVISOR

# DB 9BH,48H,1AH

OUTPUT:-

# PROGRAM 13:- WRITE A PROGRAM TO CALCULATE SUM OF A SERIES.

#BEGIN 2001H

MVI C,05

MVI B,00

LXI H,2001

DCR C

DCR C

LOOP:    INX H

ADD M

JNC SKIP

INR B

SKIP:      DCR C

JNZ LOOP

STA 3001 MOV A,B

STA 3002

HLT

#ORG 2001H

#DB 01,02,03,04,05

## OUTPUT:-